Screen Press/Screen Holder Material List

- Melamine Section, 0.75in X 16in X 8ft…..1EA

- 8 ft stick of 2X2……1EA

- 2X8 8 in long……….1EA

- 2X4 14 in long………1EA

- ¾ X ¾ X 2.1/4 in blocks….16 ea (for screen tuners)

- 4 in Hex Head Bolt (smooth shaft)….1EA

- ½ X ¾ spring

- Carriage Bolts – 3.5 in…..16 ea (for screen tuners)

- ¼ in  20 tpi nuts….17 ea  (for screen tuners and plunger)

- ¼ in wingnuts…….17 ea  (for screen tuners and plunger)

- 1.5 in deck screws….1 bx

- 2.5 in deck screws….4 ea

- 3 in deck screws….2ea

For the Magnetic Registration style press disregard itmes 3, 4, 7, and 13 above. Add:

1 stick of 1 X 3 (used in place of the 2 X 8 piece above) – cut 4 16 in sections.

Laminate 3 sections together to form the base of the backstop.  Use the last to make the sliding registration bracket that mounts on top of the base (see the supplemental video).  If using top mounted tuners in place of screen drilled tuners, add one additional layer to the laminated base.

0.5 X 1.5 in rare earth magnets…3 ea (can be obtained from 2 dead hard drives)

Mount the Y-axis magnets 5.75 inches off center and 3 inches above the table.

If using top mounted tuners in place of screen drilled tuners, mount magnets 4.25 inches above the table.  Mount X-axis magnet vertically centered on y-axis magnets and centered on the registration bracket.

Tools required:

- Saw:

I used a tablesaw, bandsaw, and circular saw.  You can use pretty much use any combination of saws that will allow you to crosscut particle board and the 2X2s,  Rip the 2X4 pieces,  and round the edges of the platens, though rounding can be accomplished with a Stanley Surform as demonstrated in the video.

- Router:										           I used a router to round/smooth the edges, but in the absence of a router, sand paper and a little elbow grease will do the job too.

- Electric drill

- Cordless drill or a screwdriver and lots of elbow grease.

- ¼ in drill bit or forsner bit

- 5/8 in Forsner bit

- 3/8 in Forsner bit

- 1/8 in drill bit

- #10 drill bit (for screen tuners)

- 3/16 in drill bit

- Hot glue gun (optional but recommended to keep from juggling parts while drilling holes and screwing pieces together).

- Pocket hole jig (optional, mounting platens to the base blocks through the top of the platen will work perfectly well.  Remember to countersink and fill/cover the holes)